# Natural-Language HIL Test Automation over ASAM XIL

Research and Architecture Plan

KPIT - Powertrain Practice / Verification & Validation      Author: Ritvik Narula      Status: Research / Proposal (no code built yet)

This document (a) frames the target architecture, (b) researches how to turn natural-language test steps into function calls, and (c) researches ASAM XIL so we can design our own library on top of it. It is a plan - nothing here is implemented yet.

# 1. Background and motivation

Today, test cases for the ECM (engine control module) are authored as NI TestStand sequences. TestStand issues the commands that drive the HIL (hardware-in-the-loop) bench and the ECU to induce signals and read responses. This works, but it has real limitations:

Rigidity - TestStand's step/sequence model is inflexible; branching, loops and data-driven logic are awkward, and the sequences are hard to reuse across programs.

Tool lock-in and skill - authoring requires the TestStand tool and trained users; the pool of people who can write and maintain sequences is small.

Scaling - a limited number of vehicle HILs must serve many programs; sequences tied to one bench do not port easily.

We want to move to the architecture in the reference diagram: the test script becomes a Python script that talks to the HIL bench through the ASAM XIL API (hardware-independent), instead of through TestStand. Crucially, we do NOT want engineers to hand-write Python - that reintroduces the skills problem. Instead, the engineer writes the test in plain English, and software turns that into the right calls.

# 2. Vision and target architecture

Three layers, top to bottom. The user only ever touches the top layer.

| Layer | Who / what | Responsibility |
| --- | --- | --- |
| 1. Readable test steps | The test engineer writes it | Near-English keyword steps, e.g. "Induce Sine Wave 1 kHz 2 V on the accelerator sensor", then "Read Variable pedal position". Authored once by a person. |
| 2. Test framework (Robot Framework) | Robot Framework engine | Maps each keyword step to the right library function and arguments - deterministically and instantly, no LLM. Robot Framework IS this layer; we do not build a parser. |
| 3. Automation library (keywords) | Python wrapper (this project) | A small, typed set of Python functions exposed to Robot as keywords. Each call drives the bench via the ASAM XIL API. |
| 4. ASAM XIL API | Standard + vendor assemblies | Hardware-independent .NET API. Our library calls it (through pythonnet) to reach the HIL bench (e.g. dSPACE) and, through it, the ECM. |
| 5. HIL bench + ECM | Hardware | Induces signals, injects faults, and reads responses from the ECM (device under test). |

The key insight: layers 3-5 are the same idea as the existing Instrument_Automation block (a thin Python wrapper over a hardware API), just with the ASAM XIL .NET API in place of raw SCPI sockets. Layer 2 is provided off-the-shelf by Robot Framework (keyword-driven), so the new work is the keyword library (layer 3) and the XIL wrapper beneath it - not a bespoke parser.

Proof-of-concept path: before wiring this to the HIL bench, we can validate the readable keyword layer against the EXISTING Instrument_Automation library (e.g. keyword "Induce Sine Wave 1 kHz 2 V" -> ts.set_function_gen_waveform(...)). Same authoring style, a library we already trust, no HIL risk. (To be done later - not now.)

# 3. Research part A - readable test steps to function calls (deterministic, no LLM)

Goal: let the engineer author a test in readable, near-English steps, and have software map each step to the right library function and arguments - reliably, fast and repeatably.

## 3.1 Why not an LLM in the run path

An LLM (function/tool calling) was considered and deliberately rejected as the run-time translator, for reasons that matter specifically when driving hardware:

Latency - calling a model on every step adds seconds per step; over a full HIL sequence with the hardware in the loop this is a large, unpredictable overhead on every run.

Dependency - a hosted model adds a network / service dependency (and data-sensitivity concerns) onto a test that must run on the bench; a local model adds heavy compute to the bench PC.

Non-determinism - the same English could translate differently between runs; test automation driving hardware must be exactly repeatable.

The translation from readable step to function call is a FIXED, finite vocabulary (our library verbs). That does not need a language model - it needs a deterministic keyword mapping. So the authoring text is done ONCE by a person; the run-time translation is a direct, instant lookup.

## 3.2 Approaches (deterministic)

(a) Robot Framework - keyword-driven (recommended). A mature, Python-based, keyword-driven test framework. Tests are written in a readable, tabular keyword syntax that reads like structured English; each keyword is backed by a Python function in a library we write. The engineer writes keywords; Robot Framework itself IS the parser that maps them to our functions - no custom translator to build. Deterministic, fast, offline; built-in HTML/XML reporting.

(b) BDD / Gherkin (Given-When-Then). The same idea in Given/When/Then sentences. Robot Framework supports this natively (it ignores the Given/When/Then/And prefixes), or tools like behave map Gherkin steps to Python step functions. Reads the most like prose, but each sentence still binds to a fixed step definition.

(c) Custom controlled-language DSL + grammar. Define our own small 'controlled English' and parse it deterministically with a grammar (e.g. Lark / pyparsing). Maximum control over the wording, but we build and maintain the parser, the reporting and the runner ourselves - most of which Robot Framework already gives us.

(d) Table / data-driven. Each test step is a row in a table (spreadsheet / CSV): a keyword column plus parameter columns, mapped to a function. Trivially deterministic and familiar (it matches how the team already captures test data), but less expressive for control flow. Robot Framework also supports this style directly (data-driven templates).

## 3.3 Recommended - Robot Framework

Robot Framework fits this project almost exactly, and it removes the need to build a bespoke 'natural-language parser' at all:

Readable authoring - keyword syntax reads like structured English; high-level keywords compose lower-level library calls, so a test step can be as plain as 'Induce Sine Wave  1 kHz  2 V'.

Deterministic, fast, offline - a keyword is a direct, instant call to a Python function; there is no per-step model inference, no network dependency, and the same step always does the same thing.

Our library ARE the keywords - the Python functions that wrap ASAM XIL are exposed to Robot as keywords directly; there is no separate translation layer to build or keep in sync.

Built-in traceability - Robot produces HTML report + log and JUnit-style XML on every run, tying each step to its result. That is exactly the requirement -> step -> result evidence the team needs for audits (ASPICE).

Fits the stack - Robot files (the readable steps) sit at the top; the Robot engine is the 'parser' layer; our keyword library is the layer below, calling ASAM XIL. It also supports BDD phrasing, data-driven tables, setup/teardown, tags and embedded arguments if we want steps to read even more like sentences.

## 3.4 Comparison

| Approach | Reads like English | Deterministic | Runtime latency | Build effort | Reporting |
| --- | --- | --- | --- | --- | --- |
| Robot Framework (keyword) | High | Yes | None | Low (write keywords) | Built-in |
| BDD / Gherkin | Highest | Yes | None | Low-Medium | Built-in / add-on |
| Custom DSL + grammar | Tunable | Yes | None | High (build it all) | Build it |
| Table / data-driven | Medium | Yes | None | Low | Via Robot |
| LLM tool calling (rejected) | Highest | No | Seconds/step | Low | Custom |

## 3.5 What it looks like

The engineer writes a readable test file; a small Python keyword library binds each keyword to an existing function. Example (against the Instrument_Automation library, as a proof of concept):

*** Settings ***

Library    InstrumentKeywords.py        # our keyword library wraps the ts.* functions

*** Test Cases ***

Measure Sine RMS

Connect Scope        oscope1    MY64520125

Configure Scope      oscope1    setup=2    channels=1

Induce Sine Wave     gen1    channel=1    frequency=1 kHz    amplitude=2 V

Enable Output        gen1    1

Capture Waveform     oscope1    1

${rms}=    Measure RMS    oscope1    1

Should Be True       5.9 < ${rms} < 6.4        # pass/fail with limits

The keyword library is thin - each keyword parses its readable arguments ("1 kHz", "2 V") and calls the real function:

# InstrumentKeywords.py  (a Robot Framework library)

import teststand_api as ts

def induce_sine_wave(alias, channel=1, frequency="1 kHz", amplitude="1 V"):

ts.set_function_gen_waveform(alias, int(channel), "SIN",

_hz(frequency), _volts(amplitude), 0.0, 0)

def measure_rms(alias, channel=1):

return ts.scope_rms(alias, int(channel))

## 3.6 Safety and traceability

Fixed vocabulary - only registered keywords (our library functions) can run; there is no open-ended interpretation.

Range / interlock checks live in the library (voltage, fault, level limits), independent of the wording - the authoring layer is convenience, not the safety boundary.

Dry-run / offline - keywords run against a fake backend first (the Instrument_Automation offline-first pattern), so a sequence can be validated with no hardware.

Robot's report/log/XML capture the full step -> result trail automatically for the audit record.

## 3.7 Proof-of-concept plan (later, not now)

Write a small Robot keyword library over the existing Instrument_Automation functions.

Author a few readable .robot test cases (sine RMS, pulse width, delay) and run them - confirming deterministic mapping and the auto-generated report.

Only then build the equivalent keyword library over the ASAM XIL wrapper and reuse the same authoring style for HIL test cases.

# 4. Research part B - ASAM XIL

ASAM XIL is an open ASAM standard that defines a generic, vendor-independent API between a test automation tool and a test bench. The same test can run on model-in-the-loop (MIL), software-in-the-loop (SIL) or hardware-in-the-loop (HIL) - the test is decoupled from the specific bench. This is exactly the 'hardware-independent test cases' goal in the reference diagram, and why it replaces bench-specific TestStand logic.

## 4.1 The ports (what you can do)

XIL exposes capability through typed 'ports'. Each port is an object with methods for one domain. The ones relevant to us:

| Port | Purpose |
| --- | --- |
| MAPort (Model Access) | Read/write simulation-model variables; capture (data acquisition); generate stimulus signals; control the simulation (start/stop/load). The workhorse port. |
| EESPort (Electrical Error Sim.) | Inject electrical faults on the wiring to the ECU - open circuit, short to ground/battery, pin-to-pin - to test diagnostics. |
| ECUMPort (ECU Measurement) | Measure ECU-internal quantities (via A2L/XCP) - values inside the controller, not just the bench model. |
| ECUCPort (ECU Calibration) | Read/write ECU calibration parameters (characteristics/maps). |
| DiagPort (Diagnostics) | Diagnostic communication with the ECU (UDS) - read DTCs, run services, clear faults. |
| NetworkPort / BusAccess | Send/receive bus frames and signals (CAN / CAN-FD / LIN / ...), generic or model-based. |

Note (from the tooling docs): the bus ports typically require at least one model port to be configured first, so MAPort is usually the foundation.

## 4.2 The object model (how the API is shaped)

The entry point is a factory that produces a vendor-specific Testbench; the Testbench hands out port and value factories; ports are configured from a *.portconfig file and then used. Typical lifecycle for a MAPort:

Load the ASAM XIL standard assemblies (ASAM AE XIL 2.x; .NET Framework 4.8 or .NET 6+).

Create the Testbench via TestbenchFactory (choose the vendor implementation, e.g. dSPACE).

From the Testbench, get the MAPortFactory and create an MAPort.

maPort.LoadConfiguration(<port>.portconfig) then maPort.Configure().

maPort.StartSimulation() (load/'go online' with the real-time application).

Read/Write model variables; set up Capturing; create and apply stimulus Signals.

StopSimulation() and Dispose() when done.

Values are strongly typed through a ValueFactory (float/int/vector/etc.), and data acquisition uses Capture / Capturing objects; stimulus uses Signal / Duration objects.

## 4.3 Calling XIL from Python (pythonnet)

The XIL API is delivered as .NET assemblies (dSPACE, NI VeriStand, Typhoon HIL and others all ship XIL implementations). From Python we bridge to .NET with the pythonnet package - this is exactly what the reference diagram shows ("Pythonnet -> TestbenchFactory"). Sketch:

import clr                                   # pythonnet

clr.AddReference("ASAM.XIL.Implementation.TestbenchFactory")

clr.AddReference("ASAM.XIL.Interfaces")

from ASAM.XIL.Implementation.TestbenchFactory.Testbench import TestbenchFactory

tbf   = TestbenchFactory()

tb    = tbf.CreateVendorSpecificTestbench("dSPACE GmbH", "XIL API", "2.x.0")

maf   = tb.MAPortFactory

port  = maf.CreateMAPort("ma")

port.LoadConfiguration("bench.portconfig"); port.Configure()

port.StartSimulation()

# port.Read(...) / port.Write(...) / capturing / signal generation ...

dSPACE provides a migration guide from the older 'HIL API MAPort (Python)' to the standard 'XIL API MAPort (.NET via pythonnet)', confirming pythonnet as the supported Python path.

## 4.4 Configuration is data

Ports are configured from vendor *.portconfig files (which model, which platform, which signals). This fits our config-as-data approach: the portconfig plus our own JSON test definitions are edited as data, not code - mirroring the Instrument_Automation config folders.

# 5. Proposed library design (our XIL wrapper)

Same philosophy as Instrument_Automation: a thin, typed Python surface over a hardware API, offline-testable, config-driven. The XIL .NET API is the 'transport' (like SCPI sockets were). Each function is exposed to Robot Framework as a keyword. Candidate 'verbs' (keywords):

| Verb (library function) | Maps to XIL | Example English |
| --- | --- | --- |
| connect_bench(config) | TestbenchFactory + MAPort LoadConfiguration/Configure | "Connect to the dSPACE bench with the diesel portconfig." |
| start / stop_simulation() | MAPort StartSimulation / StopSimulation | "Start the model." |
| set_variable(name, value) | MAPort.Write | "Set engine speed to 800 rpm." |
| read_variable(name) | MAPort.Read | "Read the reported pedal position." |
| generate_signal(name, shape, ...) | MAPort signal generation | "Induce a 1 Hz sine, 0.5-4.5 V, on the APP sensor." |
| capture(vars, duration) | MAPort Capturing | "Capture torque for 5 seconds." |
| inject_fault(pin, kind) | EESPort | "Short the AAT signal to ground." |
| read_dtcs() / clear_dtcs() | DiagPort | "Read the fault codes." |
| send_bus_signal(sig, value) | NetworkPort | "Send vehicle speed 60 km/h on CAN." |

Each verb is registered with its schema so the NL layer can target it; each performs its own range/interlock checks; each is testable offline against a fake XIL backend before it ever touches a real bench.

# 6. Repository and folder plan

This project lives OUTSIDE the Instrument_Automation block:

ASAM_XIL_Automation/

docs/    <- this research document (and future design/user docs)

src/     <- future code (NOT built yet): nl/, xillib/, configs/, tests/

Instrument_Automation/ is untouched and is reused only as the proof-of-concept target for the natural-language layer.

# 7. Risks, open questions, next steps

## 7.1 Open questions

Which XIL vendor implementation and version is on our bench (dSPACE assemblies + portconfig)?

Which ports do our test cases actually need first (MAPort + EES + Diag are the likely core)?

LLM hosting: cloud API vs. a locally-hosted / offline model (data-sensitivity and repeatability may push us to local).

How much can be deterministic 'controlled English' vs. needing a full LLM?

## 7.2 Next steps (in order, when we start building)

Prototype a Robot Framework keyword library over Instrument_Automation (safe, offline PoC).

Stand up a minimal XIL wrapper: connect, start sim, read/write one variable via pythonnet on the real dSPACE assemblies.

Grow the verb set (signals, capture, EES, diagnostics), each with offline fakes + range checks.

Point the validated NL layer at the XIL wrapper; add confirmation + traceability + reporting.

Pilot: port a slice of existing test cases onto a mini HIL bench (the diagram cites ~40% of cases as portable).

# References

ASAM XIL standard (ASAM e.V.) - https://www.asam.net/standards/detail/xil/

ASAM XIL-MA (Model Access) - https://www.asam.net/standards/detail/xil-ma/

ASAM: 'Decoupling Test Cases from Real and Virtual Test Systems with ASAM HIL API' - https://www.asam.net/index.php?eID=dumpFile&t=f&f=1456

dSPACE - ASAM XIL API - https://www.dspace.com/en/inc/home/applicationfields/portfolio/xil_api.cfm

dSPACE - HIL API MAPort Python to XIL API .NET (pythonnet) Migration Guide - https://www.dspace.com/en/inc/home/support/kb/supkbspecial/kbta/hilapimaportpythonxilapimig.cfm

NI - ASAM XIL Generic Simulator Interface (VeriStand) - https://www.ni.com/docs/en-US/bundle/veristand/page/asam-xil-interface.html

NI Community - Example Python Scripts for ASAM XIL API - https://forums.ni.com/t5/NI-VeriStand-Add-Ons-Documents/Example-Python-Scripts-for-ASAM-XIL-API/ta-p/3776234

Typhoon HIL - ASAM XIL API Python Guide - https://www.typhoon-hil.com/documentation/typhoon-hil-software-manual/concepts/asam_xil_api_python_guide.html

tracetronic ecu.test - Generic XIL API (ports) - https://docs.tracetronic.com/help/ecu.test/en/_build/Tools/Software-interfaces/GENERIC-XIL-API.html

Robot Framework (official site) - https://robotframework.org/

Robot Framework User Guide - https://robotframework.org/robotframework/latest/RobotFrameworkUserGuide.html

Robot Framework - keyword-driven testing (guide) - https://www.apriorit.com/qa-blog/569-keyword-driven-testing-robot-framework

Gherkin / BDD style with Robot Framework - https://dev.to/leading-edje/gherkin-and-robot-framework-5oe