# TestStand Bench API - Test Developer Guide

This guide is for the TEST DEVELOPER building a TestStand sequence against the combined bench API (teststand_api.py). It explains the flow of logic - which functions to call, in what order, and why - and walks through the real use cases step by step, each with an objective, the exact calls, and the expected result.

Every function takes and returns simple values (string, number, boolean, list), so you bind them straight to sequence variables. Instruments are addressed by an alias you choose; the module picks the scope driver or the generator driver for each from its *IDN?.

Script location: scripts/real/teststand_api.py

# Index

Right-click here and choose Update Field (or select and press F9).

# 1. The big picture

One module drives the whole bench - oscilloscopes and function generators together. You connect each instrument under an ALIAS (for example "oscope1", "funcgen1"), and from then on every call takes that alias first. Scope calls (configure_scope, capture_scope, scope_*) address scopes; generator calls (configure_function_gen, function_gen_*) address generators. The module chooses the right driver for each instrument automatically from its identity string.

Two habits keep sequences safe and portable:

Name instruments by SERIAL, not IP. connect_matching(alias, serial) always opens the same physical instrument, whatever link-local address it gets.

Enabling a generator output drives real hardware. Configuring a waveform never turns an output on - you call function_gen_output_on() deliberately, and all_function_gen_outputs_off() in Cleanup.

# 2. Bench setup (do this once, before any use case)

Three instruments on a link-local HIL bench, all reached over SCPI port 5025.

## 2.1 Physical setup

With every instrument powered OFF, connect each one to the Ethernet switch, then connect the switch into the HIL bench network.

Power on the instruments.

On the Tektronix oscilloscope, set the Socket Server to Terminal mode on port 5025 (Utility -> I/O -> Socket Server). The Keysight scope and the AFG already use 5025.

## 2.2 Software bring-up

import teststand_api as ts

ts.detect_bench(5025)          # find + open every instrument on port 5025

# -> ["oscope1", "oscope2", "funcgen1"]

detect_bench identifies each instrument by *IDN? and opens it with the right driver. To target one device by serial under your own alias instead, use connect_matching:

ts.connect_matching("oscope1", "MY64520125", 5025)

## 2.3 The bench

funcgen1 - Tektronix AFG31102, serial B011788 - the stimulus (2 channels).

oscope1 - Keysight DSO-X 3034G, serial MY64520125.

oscope2 - Tektronix MSO24, serial C017370.

# 3. The common flow of logic

Almost every use case is the same shape. Fill in the measurement or the save step for the job at hand.

Setup:    connect_matching(alias, serial, 5025)      open the instrument by serial

configure_scope(alias, setup, channels)     apply + verify the scope setup

(configure_function_gen / set_function_gen_waveform, if a stimulus is needed)

Main:     (function_gen_output_on, if driving a signal)

capture_scope(alias, channels)              read the record

scope_rms / scope_pulse_width / scope_mean  measure  <- LIMITS here

save_scope_png / save_scope_csv             save (labeled by alias)

Cleanup:  all_function_gen_outputs_off()

disconnect_all()

When the signal is supplied by something else (the ECM, a bench supply), you skip the generator steps entirely - see the DC use case.

# 4. Use cases (step by step)

Each use case gives the objective, the exact calls with parameters, and the expected result. They assume the bench setup in section 2 is done.

## 4.1 RMS voltage

Objective: Measure the true RMS voltage of a signal on a scope channel.

Steps (after the bench setup):

ts.connect_matching("oscope1", "MY64520125", 5025)     # the Keysight scope

ts.configure_scope("oscope1", "signal_view", "1")

ts.set_function_gen_waveform("funcgen1", 1, "SIN", 1000, 10, 5)  # drive a 1 kHz sine

ts.function_gen_output_on("funcgen1", 1)

ts.capture_scope("oscope1", "1")

rms = ts.scope_rms("oscope1", 1)                       # put LIMITS on this step

Expected result: rms ~ 6.14 V - the true RMS of a 0-10 V, 1 kHz sine.

## 4.2 Pulse width (positive and negative)

Objective: Measure how long a square / pulse signal stays high and how long it stays low.

Steps (after the bench setup):

ts.connect_matching("oscope2", "MSO24", 5025)          # the Tektronix MSO24

ts.configure_scope("oscope2", "signal_view", "1")

ts.set_function_gen_waveform("funcgen1", 2, "SQU", 1000, 10, 5)  # 1 kHz square

ts.function_gen_output_on("funcgen1", 2)

ts.capture_scope("oscope2", "1")

pos = ts.scope_pulse_width("oscope2", 1)               # high time

neg = ts.scope_pulse_width_negative("oscope2", 1)      # low time

Expected result: pos ~ 0.500 ms and neg ~ 0.500 ms - a 1 kHz square is 1.000 ms period, 50% duty.

## 4.3 Capture and save a waveform

Objective: Capture a channel and save the waveform to CSV + PNG, labeled by scope name.

Steps (after the bench setup):

ts.connect_matching("oscope1", "MY64520125", 5025)

ts.configure_scope("oscope1", "signal_view", "1")

# drive a 1 kHz sine on funcgen1 CH1, output on

ts.capture_scope("oscope1", "1")

ts.save_scope_csv("oscope1", "ch1.csv")   # -> results/oscope1_ch1.csv

ts.save_scope_png("oscope1", "ch1.png")   # -> results/oscope1_ch1.png

Expected result: The captured waveform, saved. The alias is written into both the file name and the plot title, so oscope1's and oscope2's results never mix up.

## 4.4 Route one generator into two scopes

Objective: Drive two different waveforms from one generator, each into a different scope, and capture both.

Steps (after the bench setup):

ts.connect_matching("oscope1", "MY64520125", 5025)

ts.connect_matching("oscope2", "MSO24", 5025)

ts.connect_matching("funcgen1", "B011788", 5025)

ts.configure_scope("oscope1", "signal_view", "1,2")

ts.configure_scope("oscope2", "signal_view", "1")

ts.set_function_gen_waveform("funcgen1", 1, "SIN", 1000, 10, 5)  # -> oscope1 CH1

ts.set_function_gen_waveform("funcgen1", 2, "SQU", 1000, 10, 5)  # -> oscope2 CH1

ts.function_gen_output_on("funcgen1", 1)

ts.function_gen_output_on("funcgen1", 2)

ts.capture_scope("oscope1", "1"); ts.save_scope_png("oscope1", "ch1.png")

ts.capture_scope("oscope2", "1"); ts.save_scope_png("oscope2", "ch1.png")

Expected result: oscope1 captures the sine (RMS ~ 6.14 V), oscope2 captures the square (Vpp ~ 10.3 V). Files: results/oscope1_ch1.* and results/oscope2_ch1.*.

Runnable file: scripts/real/teststand_named_demo.py

## 4.5 Read a flat DC voltage (no generator)

Objective: Read a steady DC voltage that is already present on a scope channel - supplied EXTERNALLY by the ECM, a bench supply, or a battery. Nothing is generated.

Steps (after the bench setup):

ts.connect_matching("oscope1", "MY64520125", 5025)

ts.configure_scope("oscope1", "dc_read", "1")     # DC coupling, AUTO trigger

ts.capture_scope("oscope1", "1")

level = ts.scope_mean("oscope1", 1)               # the DC level

ripple = ts.scope_vmax("oscope1", 1) - ts.scope_vmin("oscope1", 1)

Expected result: level = the measured DC voltage (e.g. ~14.0 V for an automotive line). The dc_read setup covers roughly +/-20 V; drop the vertical scale for small logic levels, raise it for higher voltages.

Runnable file: scripts/real/teststand_dc_level.py

## 4.6 Slow single-shot: a 1 mHz triangle over a 20-minute run

Objective: Induce a 1 mHz triangle (period 1000 s) into the ECM for 20 minutes and capture the whole run as one single-shot record.

Steps (after the bench setup):

ts.connect_matching("funcgen1", "B011788", 5025)

ts.connect_matching("ecm_scope", "MSO24", 5025)   # Tektronix: 120 s/div > Keysight max

ts.configure_function_gen("funcgen1", "triangle_1mhz", "1")

ts.configure_scope("ecm_scope", "ecm_20min", "1")

ts.function_gen_output_on("funcgen1", 1)

ts.capture_scope("ecm_scope", "1", single=True, timeout_s=1380)  # arms once, blocks ~20 min

ts.save_scope_png("ecm_scope", "ecm_triangle.png")

Expected result: One 1200 s record (~1.2 triangle cycles). single=True arms a single acquisition with an AUTO trigger (starts immediately, little delay) and blocks until the record fills. The Tektronix scope is REQUIRED: 120 s/div exceeds the Keysight's 50 s/div ceiling.

Runnable file: scripts/real/teststand_ecm_triangle.py

# 5. Function reference (the calls used above)

Parameters and return values for the functions the use cases use. Full reference: the teststand_api (bench) User Manual.

## 5.1 Connect

connect_matching(alias: str, match: str, port: int = 5025, subnet: str = "") -> str

Discover, then open the instrument whose *IDN? contains `match` (usually its serial) under YOUR alias, with the right driver.

Parameters:

alias (str) - your name for the instrument.

match (str) - a substring of the target's identity - typically its serial.

port (int) - SCPI port. Default 5025.

Returns: str - the *IDN?, or "" if nothing matched.

detect_bench(port: int = 5025, subnet: str = "") -> list[str]

Detect and open every instrument on the bench with the right driver.

Returns: list of str - the aliases created.

## 5.2 Scope: configure and capture

configure_scope(alias: str, setup: str = "bench_full", channels: str = "1,2", duration_s: float = 0.0) -> bool

Apply a named scope setup and verify every setting read back.

Parameters:

setup (str) - a setup file name, e.g. "signal_view", "dc_read", "ecm_20min".

channels (str) - "1" or "1,2".

duration_s (float) - the capture window, total seconds across the screen (s/div = duration_s / 10). The window is NOT in the config, so pass this - each config's comment gives the value (WINDOW: duration_s=...). See 5.7.

Returns: bool - True only if every setting verified.

capture_scope(alias: str, channels: str = "1", points: int = 0, single: bool = False, timeout_s: float = 120.0) -> bool

Read one or more channels into memory for the measurement and save steps.

Parameters:

single (bool) - False forces a fresh record and reads it (the normal choice). True arms ONE real acquisition and blocks until it completes - for a slow single-shot; set timeout_s longer than the record.

timeout_s (float) - with single=True, how long to wait for completion.

Returns: bool - True if every requested channel returned data.

Errors: Raises TimeoutError if single=True and the acquisition never completes.

## 5.3 Scope: measurements (put LIMITS here)

scope_rms(alias, channel: int = 1) -> float

True RMS volts (any shape).

Returns: float - volts.

scope_mean(alias, channel: int = 1) -> float

Average volts - the DC level.

Returns: float - volts.

scope_vmax / scope_vmin(alias, channel: int = 1) -> float

Maximum / minimum volts (their difference is Vpp).

Returns: float - volts.

scope_pulse_width(alias, channel: int = 1) -> float

Positive (high) pulse width, seconds.

Returns: float - seconds. 0.0 if flat.

scope_pulse_width_negative(alias, channel: int = 1) -> float

Negative (low) pulse width, seconds.

Returns: float - seconds. 0.0 if flat.

scope_delay(alias, source1=1, source2=2, edge1="rising", edge2="falling", direction="forward") -> float

Delay between two channels, measured by the scope and shown on its screen (no capture needed). Times the chosen edge on source1 to the chosen edge on source2; edge1/edge2 are "rising" or "falling" in any combination.

Returns: float - seconds (NaN if the scope has no valid measurement).

## 5.4 Scope: save

save_scope_png(alias: str, path: str) -> str

Save a PNG of the captured data; the alias goes in the file name AND the plot title.

Returns: str - path(s) written. Raises RuntimeError if nothing captured for that scope.

save_scope_csv(alias: str, path: str) -> str

Save the captured data as CSV, alias in the file name.

Returns: str - path(s) written.

## 5.5 Generator

set_function_gen_waveform(alias: str, channel: int = 1, shape: str = "SIN", frequency: float = 1000.0, amplitude: float = 1.0, offset: float = 0.0, duty_cycle: float = 0.0) -> bool

Set one generator channel's waveform directly and verify it. Does NOT enable the output.

Parameters:

shape (str) - "SIN", "SQUare", "RAMP", "PULSe", "DC", ...

frequency (float) - Hz.

amplitude (float) - Vpp.

offset (float) - volts.

Returns: bool - True only if verified.

configure_function_gen(alias: str, setup: str = "sine_1k", channels: str = "1") -> bool

Apply a named generator setup and verify it. Does NOT enable the output.

Returns: bool - True only if verified.

function_gen_output_on(alias: str, channel: int = 1) -> bool

Switch a generator channel's output ON. Drives real hardware - call it deliberately.

Returns: bool - True.

## 5.6 Cleanup

all_function_gen_outputs_off() -> bool

Switch every output OFF on every connected generator.

Returns: bool - True.

disconnect_all() -> bool

Close every session.

Returns: bool - True.

## 5.7 Config parameters - what each field expects

A config is one JSON file (a single scope or generator setup) in the configs/ folders. Plain numbers in the scope's native units - no unit strings. Leave a field null to leave that setting untouched. Scope-wide fields:

| Field | Expected value | What it means |
| --- | --- | --- |
| record_length | integer (points) | Number of samples in the record, e.g. 10000. |
| trigger_position | 0-100 (percent) | Where the trigger sits across the screen; 10 = 10% from the left. |
| sample_rate | number (S/s) or null | Usually null (the scope works it out); set a number only to force a rate. |
| trigger_source | "CH1" or "CH2" | Which channel the trigger watches. |
| trigger_type | "EDGE" | Trigger type (only EDGE supported). |
| trigger_slope | "RISE" or "FALL" | Which edge fires the trigger. |
| trigger_level | volts, or "half_battery" | Crossing level. "half_battery" = battery_voltage / 2. |
| trigger_mode | "AUTO" or "NORMal" | AUTO free-runs; NORMal waits for the real trigger (use for a one-time event, with capture single=True). |
| battery_voltage | volts, or null | Only used to resolve "half_battery". |
| acquire_mode | "SAMple" | Acquisition mode. |
| measurement | "rms"/"delay"/... or null | A note of what the test records; metadata only. |

Per-channel fields, under channels."1" / channels."2":

| Field | Expected value | What it means |
| --- | --- | --- |
| input_coupling | "DC" or "AC" | Input coupling. |
| vertical_scale | number (V/div) | Volts per division, e.g. 5.0. |
| vertical_position | number (divisions) | Vertical position in divisions (Tektronix; Keysight uses offset). |
| offset | volts, or null | Vertical offset in volts. |
| termination | 1000000 or 50 | Input impedance in ohms (1 MOhm or 50 Ohm). |
| bandwidth | number (Hz) or null | Input bandwidth limit (Tektronix). |

The capture window is NOT a field: pass duration_s to configure_scope (total seconds; s/div = duration_s / 10). Each config's comment lists the value (WINDOW: duration_s=...).

Generator (AFG) configs use their own fields, under channels."1":

| Field | Expected value | What it means |
| --- | --- | --- |
| shape | "SIN"/"SQUare"/"RAMP"/"PULSe"/"DC" | Waveform shape. |
| frequency / amplitude / offset | Hz / Vpp / volts | Sine/ramp/square basics. |
| duty_cycle | percent | PULSe/SQUare high-time duty; RAMP rise symmetry (50 = triangle). |
| impedance | 50 or "INFinity" | Output load - match the ECM/scope input (high-Z = INFinity). |
| pulse_rate / pulse_width | seconds | Pulse period / high-time (pulse-by-levels). |
| pulse_high_voltage / pulse_low_voltage | volts | Pulse levels. Use these OR amplitude+offset, not both. |

# 6. Pass/fail, errors, and output safety

A function that returns False = the TEST FAILED (a setting did not verify, a channel returned no data). Bind these boolean returns to a pass/fail step.

A function that RAISES = something is BROKEN (an instrument unreachable, a channel never captured, a single-shot that timed out). TestStand shows a step error.

Measurements go on Number steps with LIMITS - that is where the signal actually passes or fails.

Enabling an output drives hardware. Configure never enables it; call output_on deliberately, and all_function_gen_outputs_off() in Cleanup.

capture_scope: single=False (the default) is right for a signal that is already present. single=True is for a deliberate single-shot you start and wait for (for example the 20-minute triangle in use case 4.6) - give it a timeout longer than the record.

# 7. Runnable example files

Each use case has a ready-to-run file - just run it, no glue code:

scripts/real/teststand_named_demo.py     one generator into two scopes (use case 4.4)

scripts/real/teststand_dc_level.py       read a flat DC voltage (use case 4.5)

scripts/real/teststand_ecm_triangle.py   1 mHz triangle, 20-minute capture (use case 4.6)

configure_detected_bench(5025) is a one-call bring-up: it detects the whole bench, configures every instrument, and fills get_bench_report() with the evidence - a quick smoke test before a real sequence.

# 8. Common problems

No instruments detected / connect_matching returns ""

Run scan_bench() to see what is actually present and its serials. Check the physical link and that each SCPI socket is on the port. Last-known IPs are cached, so a good first run keeps working.

configure_scope / configure_function_gen returns False

A setting did not read back. get_scope_config_report() / get_function_gen_config_report() shows which line - often the instrument quantized a value (e.g. a Tek snapping sample rate or timebase). The capture still runs.

capture_scope returns no data

The channel was not displayed / not acquiring, or (single=True) the acquisition never completed. Check scope_captured_channels(); widen the timeout for a slow single-shot.

A measurement raises "No captured data"

That channel was not in the last capture_scope() for that scope. Capture it first.

The generator output nothing

Configuring a waveform does not enable the output. Call function_gen_output_on().

A 20-minute (ecm_20min) capture fails on the Keysight

120 s/div exceeds the Keysight's 50 s/div limit. Use the Tektronix MSO24 for it.